- U-easytech                   STM32-X64-MC-03R/UET

- STM32-X64-MC-03R/UET BOARD

Description:

The STM32-X64-MC-03R/UET board has been designed as a complete demonstration and development platform for the ARM cortex-M0 core-based STM32F030R8T6 microcontroller. It features one I2Cs, two SPIs, two USARTs, one 12-bit 16 channels ADCs, internal 8-KByte Data SRAM and 64-KByte Flash, JTAG SWD mode debugging support. This evaluation board can be used as the reference design for user application development but it is not considered as a final application. With the SWD interface you can use debugger STLINKIII or JLINKV8 to test your program.

- Features:

- MCU: STM32F030R8T6 (64pin)

- Core work at up to 48M frequency

- 64KB Flash and 8KB RAM

- l 2.4’ TFT LCD

- 4 User Leds, 5 User Keys

- 1 Power Key, 1 Reset Key

- 1 Power Led

- 1 DV 5V power interface

- 1 I2C EEPROM 24C02

- 1 SPI SD card interface

- 1 AD channel with Pointer

- l RTC with backup battery interface

- With motor control connector

- l JTAG Debug port

- l ALL I/O expansion

Order NO: STM32-X64-MC-03R /UET

System description：

JTAG PORT

Rower Key

Power Led

DV 5V

USB Power

SWD port

All IO pin

STM32F030R8T6

Core board

Boot choose

SPI SD card

I2C eeprom

Motor control port

2.4 串行 TFT

4 User Ledss

AD Pointer

Reset Key

5 user keys

- U-easytech                   STM32-X64-MC-03R/UET

- STM32-X64-MC-03R/UET 开发板

简介:

STM32-X64-MC-03R/UET开发板是一块基于CORTEX-M0内核的STM32F030R8T6微处理器的开发平台。STM32F030R8T6有1个I2C,2个SPI,2个USART, 1个12位16通道ADC转换器，内部有8KB的Data SRAM，64KB的FLASH,支持JTAG SWD模式调试。此评估平台可作为用户应用开发时做参考，而未作最终产品应用考虑。用户可通过SWD用STLINK或JLINK仿真下载程序。

- 特点:

- CPU:STM32F030R8T6 (64pin)

- 最大工作频率：48M

- 64K FLASH,8K RAM

- 一个2.4  TFT 液晶

- 4个用户指示灯，5个用户按键

- 1个电源开关，一个复位键

- 一个电源指示灯

- 一个DV5V输入接口

- 一个24C02 存储芯片

- 一个AD电位器

- 电池接口

- 电机控制接口

- 一个SPI SD卡接口

- 一个SWD下载口

- 所有IO引出

订单编号: STM32-X64-MC-03R /UET